NetContinuum

Web Application Hacking Seminar

Part III

Setting Up the Basics

Stefan Drege

Sr. Manager, International Partner Enablement

SDrege@NetContinuum.com

© 2002-2004 NetContinuum, Inc.  All Rights Reserved

The content of this guide is protected by international copyright law and may not be reproduced without permission.

This guide is provided as a sample procedure and is provided for informational use only.   NetContinuum, inc. reserves the right to make changes to this document, and assumes no responsibilities or liability, for errors, omissions, in this guide.

NetContinuum and Web Security Gateway are either registered trademarks or trademarks of NetContinuum, Inc.  All other trademarks and registered trademarks are property of their respective owners.

Introduction	3

What This Document Covers	3

Seminar Setup Activities	3

Overview	3

Step 1 – Creating the BadStore.net Web Server	3

Step 2 – Preparing the Presenter’s Workstation	3

Step 3 – Preparing the NC-1000	3

Last Step – Validate Connectivity	3

# Introduction

The NetContinuum Web Hacking Seminar series of documents is designed to help the NetContinuum Partner perform a smooth seminar.  There are four guides:

- Part I – Seminar Objectives

  - Provides a philosophical overview of the NetContinuum Web Hacking Seminar process

- Part II – What You Will Need

  - What components are required to conduct the seminar, and how to configure these components

- Part III – Setting Up the Seminar (This Document)

  - The steps to take to set up and prepare for the seminar.

- Part IV – Running the Seminar

  - The steps – what to do and what to emphasize – during the seminar itself.

These documents are targeted at the Systems Engineer that has the responsibility to prepare for and conduct un-aided NetContinuum Web Hacking Seminar.

This document, “Part III – Setting Up the Environment” provides detailed instructions for the initial setup process.  After performing the steps outlined in this document, the network will be ready for you to start your Web Application Hacking Seminar.  Following these directions closely will provide the basis for a successful seminar.

In order for the NetContinuum Web Application Hacking Seminar objectives to be met (see document entitled “Part I – Seminar Objectives”), the seminar network environment must remain consistent.  This document builds upon the assumed network configuration outlined in the document entitled “Part II – What You Will Need”.

# What This Document Covers

This document provides the detailed steps required to configure the entire seminar attack demonstration environment.  This includes the front-end laptop/workstation, the NC-1000, and the back-end web server configurations.

The NetContinuum Evaluation Components

Front-End System

Windows 2000/XP Microsoft Internet Explorer®

NetContinuum Management GUI

Paros Proxy

Syslog Daemon

Back-End System

Web Server Booted

from Trinix CD

Supplied By NetContinuum

Hub or Switch

NC-1000 in “One-Armed” Configuration

192.168.1.254

192.168.1.202

IF2 - 0.0.0.0

PIF – 192.168.1.201

IF1 – 192.168.1.200

VIP – 192.168.1.203

Mgmt – 192.168.1.100

# Seminar Setup Activities

Preparation of the network and components within that network are key to making your Seminar successful.

- Booting a workstation from the NetContinuum Badstore.Net CD

- Configuring the addresses and network cabling.

- Installing the PAROS proxy software to the presenter’s workstation.

- Configuring the NC-1000 for Phase II of the Seminar

## Overview

The seminar is presented in two phases.  The first 1 hour phase shows the presenter launching attacks directly against the BadStore.net application.  During the second phase (also 1 hour) the presenter launches same or similar attacks into the application via the NC-1000.  To accommodate this, the presenter’s workstation has connectivity to the server directly, and the NC-1000 front end VIP.

It is assumed that only the presenter’s workstation will be projected to the screen.

## Step 1 – Creating the BadStore.net Web Server

Part of the Web Hacking Seminar kit includes the BadStore application.  It is supplied as a bootable CD image.

- Create the Boot CD:

  - The ISO image is located in the following subdirectory:

    - <CDROM>:\SeminarSoftware\BadStoreSite\BadStore_123.ISO

  - Create a writeable CD from this image.  This will create the bootable CD.

- Boot the Web Application Server

  - Insert this CD to your back-end system/laptop, and have it boot from that CD. When the host system is booted from the CD apache and the BadStore.net application are brought up using Trinux and RAMDisk.  Thus the host system is not corrupted.

- Set Up the Addressing

  - During initialization, the BadStore.net CD will pause looking for a DHCP assigned address.  Let the boot process complete and override any assigned address if present:

  - Upon boot completion, hit the Enter key to access the console

  - Enter “ifconfig eth0 up 192.168.1.202” at the command prompt.  This will set the IP address of the server.

  - Note: If available, a VMWare® virtual system may be used to host this site.

Make Sure that You Can Access this site from the presenters’ workstation!

## Step 2 – Preparing the Presenter’s Workstation

The presenter will be launching attacks and using the NC-1000 management GUI during phase II of the seminar.  Some software will have to be installed on the presenter’s workstation, therefore.

- The presenter’s workstation IP address should be set to 192.168.1.254

- The NetContinuum GUI should have been installed already.  The Seminar Steps in this guide assume NetContinuum OS and GUI of 4.3.x version.

- The PAROS proxy must be added to the client workstation.

  - The PAROS proxy requires Java Runtime 1.4.0 or higher.  It is recommended to use the JRE 1.4.2.06 version supplied on the Seminar Kit CD under the “<CDROM>:\SeminarSoftware\JRE” subdirectory.  Install the software according to the instructional dialogs using defaults.

  - The PAROS software can be found on the Web Hacking Seminar Kit CD under the “<CDROM>:\SeminarSoftware\Paros\” subdirectory.  Install the software according to the instructional dialogs using defaults.

In order to clarify and simplify the presentation – particularly in Phase II, some entries will be added to the client workstation HOSTS file:

  - Into C:/WINDOWS/SYSTEM32/DRIVERS/ETC/HOSTS file, add the following:

    - 192.168.1.202	Direct.BadStore.net

    - 192.168.1.203	Protected.BadStore.net

  - These host names are used in the step-by-step guide that follows.

## Step 3 – Preparing the NC-1000

The NC-1000 will be used during Phase II of the seminar.  This is one in which the same attacks are attempted, but blocked by the NC-1000 configuration.

Highlights of the configuration include:

  - A one-armed configuration such that the physical BadStore.net server and the NC-1000 VIP are accessible from the browser without network changes.  As such, the assumed initial-addressing within the NC-1000 is assumed to be:

    - NC-1000 System Name: NC-1000

    - Management Port – 192.168.1.100

      - Default gateway – 192.168.1.254

  - The supplied XML NC-1000 configuration file should be merged into the configuration.  This configuration has been pre-defined as per the requirements of the BadStore application. This file is located at:

    - <CDROM>:\SeminarSoftwarre\NC-Config\BadStore.xml

Your NC-1000 configuration should now have the following properties:

  - VSite – Datacenter

  - Web Application – Protected.BadStore.net

  - VIP – 192.168.1.203

  - ServerGroup – BadStoreServers

  - Server – Server202

## Last Step – Validate Connectivity

The last part of setting up is to ensure you have connectivity to the application from the presenter’s workstation browser.

- Use your browser to access http://direct.BadStore.net.  The target site will appear.

- Use your browser to access http://protected.BadStore.net.  The target site will appear, but access will have been routed through the NC-1000.

The network is now ready for your Web Application Hacking Seminar!

As the next step, proceed to the document entitled “Part IV – Running the Seminar”.

**** Remember to Rehearse the Script! ****

(End of Document)